Conveyor

User Manual

SAFETY AND OPERATION INSTRUCTIONS

IMPORTANT: Read safety rules and instructions carefully before operating equipment.

CAUTION

BEFORE, DURING AND AFTER OPERATION

—Before using, make sure that the main body of the engine is added lubricating oil, and the engine fuel tank adds gasoline.

—Besides, there is a hydraulic tank beside the engine, so we need to inject a certain amount of hydraulic oil to ensure that the oil level is higher tha n the oil outlet.

SAFETY

• Wear safety equipment when operating conveyor

• Goggles

• Hard hat

• Gloves

• Etc.

• Never operate conveyor under influence of drugs or

alcohol.

• Keep area under conveyor clear of obstructions

• Do not allow kids to operate or play around conveyor

• Do not stand under conveyor while it is running

• Do not put hands on running belts or chains

• Never run without guards in place

TOWING

• Requires a 2 inch ball for towing

• Make sure conveyor is empty no debris or material

that may fall off when towing

• Use safety chains while traveling

• Use safety flags to properly mark conveyor while traveling

on roads or highways

• Raise the conveyor bed by manual winch to a height higher,

•This conveyor is only suitable for yard or close to yard so towing great distances are not recommended

BEFORE OPERATION

• Have conveyor on level ground

• Check gas and oil levels

• Be sure conveyor is stable

• Block wheels to prevent rolling

• Make sure area under conveyor is clear of any

obstructions

• Lower support legs and secure with locking bolts

• Look over conveyor to assure everything is in proper

working order.

• Make sure belt or chain is clear for easier start up.

MAINTENANCE AND ADJUSTMENTS

• Do not over grease the bearings

• Grease bearings with single pump after 40 hours of use

• Keep sliders greased

• Keep pivot point at height adjustment greased

• Engine - See engine manual

WARRANTY

• Motor - 1 years

• Conveyor - 6 months against manufacturer defects

•Warranty is for 6 months from purchase date and is for replacement parts only (no labour charges apply ) and it is a back to our depot warranty which means if you did have a failure which we could not remedy on site you would need to return to our depot where we would repair or replace the conveyor if needed

ADJUSTING BELT/CHAINOPERATING CONVEYOR

• Set conveyor at desired speed

• Make sure conveyor is running properly before loading

• While conveyor is running be sure there is nothing

underneath the drop point.

• Depending on speed you are loading the conveyor

determines the speed you set the belt or chain.

• Be sure not to block opening of the conveyor

HEIGHT ADJUSTMENT

• To use manual winch to adjust conveyor height, see follow picture.

Assemble:

According follow drawing to finish assembling conveyor.

First step:

Assembly conveyor transport

Second step:

Assembly frame parts

Third step:

Assembly bar

Fourth step:

ssembly wheels/belt

• How do you adjust tracking

• If belt or chain loosens adjust from

top of conveyor (do not adjust from

bottom)

• Use ¼ turn at a time until belt or

chain is realigned.

• To lower conveyor take manual winch handle and turn slowly.

• Note: Turning release valve too quickly could cause

conveyor to drop too fast and may cause injury.

TECHNICAL SPECIFICATIONS

| Engine brand | Briggs & Stratton |
| --- | --- |
| Horse power | 6.5HP Petrol 4 Stroke Engine |
| Engine start | Recoil |
| Max.Height | 2.8M |
| Belt Width/thickness | 300mm/5mm |
| Belt Travel Speed | 0.7m/s |
| Hydraulic Oil Tank Capacity | 12L |
| Hydraulic driven Pump | 5.3 Gallon Per Minute |
| Tire | 5.7-8 |
| Height adjust | Use Manual winch adjust. |
| Net Weight | 285kg |
| Package Size | 2250*600*730mm |
| Color | Orange |