OWNER’S/OPERATOR’S MANUAL

BACKPACK BRUSHCUTTER

Introduction	Attention Statements

The 35-series of hand power tools has been designed and built to de- liver superior performance and reliability without compromise to quality, comfort, or durability.

high performance engines represent the leading edge of 2-cycle engine technology, delivering excep- tionally high power from remarkably low displacement and weight. As an owner/operator, you’ll soon discover for yourself why is simply in a class by itself!

IMPORTANT!

The information contained in this manual describes units available at the time of publication.

Inc. reserves the right to make changes to products without prior notiﬁcation, and without obligation to make alterations to units previously manufactured.

Contents	PAGE

Attention Statements	2

General Safety Instructions	3

Checking Unit Condition	3

Unit Description	5

Speciﬁcations	5

Assembly	6

Mixing Fuel	9

Starting the Engine	10

Starting a Flooded Engine	10

Stopping the Engine	11

Engine Idle Adjustment	11

Using a Brushcutter Blade	12

Cutting Grass (trimmer head)	13

Maintenance	14

Spark Arrester	15

Long Term Storage	16

Blade Sharpening	16

Troubleshooting Guide	17

Throughout this manual are “Special Attention” statements.

WARNING!

A statement preceded by the trian- gular attention symbol and the word “WARNING” contains information that should be acted upon to prevent serious bodily injury.

CAUTION!

A statement preceded by the word “CAUTION” contains information that should be acted upon to prevent mechanical damage.

IMPORTANT!

A statement preceded by the word “IMPORTANT” is one that possesses special signiﬁcance.

NOTE:

A statement preceded by the word “NOTE” contains information that is handy to know and may make your job easier.

Read and follow this operators manual. Failure to do so could result in serious injury.

Wear eye and hearing protection at all times during the operation of this unit.

Keep bystanders

at least 50 feet (15 m) away during operation.

Beware of thrown or ricocheted objects.

Beware of blade thrust. A jammed blade can cause the brushcutter to jerk suddenly and may cause the operator to lose control

of the unit.

IMPORTANT!

The operational procedures described in this manual are intended to help you get the most from this unit as well as to

protect you and others from harm. These procedures are guidelines for safe operation under most conditions, and are not intended to replace any safety rules and/or laws that may be in force in your area. If you have questions regard- ing your power tool, or if you do not understand something in this manual, your dealer will be glad to assist you. You may also contact , Inc. at the address printed on the back of this manual.

General Safety Instructions	Checking Unit Condition

### Work Safely

WARNING!

Use Good Judgment

NEVER operate the engine when transporting the unit.

NEVER operate the engine indoors! Make sure there is always good ven- tilation. Fumes from engine exhaust can cause serious injury or death.

ALWAYS use the proper cutting tool for the job.

ALWAYS stop the unit immediately if it suddenly begins to  vibrate  or shake. Inspect for broken, missing or improperly installed parts or attach- ments.

NEVER extend trimming line beyond the length speciﬁed for your unit.

ALWAYS keep the unit as clean as practical. Keep it free of loose vegeta- tion, mud, etc.

ALWAYS hold the unit ﬁrmly with both hands when cutting or trimming, and maintain control at all times.

ALWAYS keep the handles clean.

ALWAYS disconnect the spark plug wire before performing any maintenance work.

ALWAYS, if a blade should bind fast in a cut, shut off the engine immedi- ately. Push the branch or tree to ease the bind and free the blade.

Brushcutters operate at very high speeds and can do serious damage or injury if they are misused or abused. Never allow a person without training or instruction operate this unit!

WARNING!

Never make unauthorized attach- ment installations.

### Stay Alert

You must be physically and mentally ﬁt to operate this unit safely.

NEVER operate the unit with the cutting attachment shield or other protective devices (harness, ignition switch, blade retention clip, etc.) removed!

WARNING!

A cutting attachment shield or other protective device is no  guarantee of protection against ricochet. YOU MUST ALWAYS GUARD AGAINST FLYING DEBRIS!

WARNING!

Never operate power  equipment of any kind if you are tired or if you are under the inﬂuence of alcohol,

drugs, medication or any other sub- stance that could affect your ability or judgement.

WARNING!

Minimize the Risk of Fire

NEVER smoke or light ﬁres near the unit.

ALWAYS stop the engine and allow it to cool before refueling. Avoid overﬁlling and wipe off any fuel that may have spilled.

ALWAYS inspect the unit for fuel leaks before each use. During each reﬁll, check that no fuel leaks from around the fuel cap and/or fuel tank.  If fuel leaks are evident, stop using  the unit immediately. Fuel leaks must be repaired before using the unit.

ALWAYS move the unit to a place well away from a fuel storage area or other readily ﬂammable materials before starting the engine.

NEVER place ﬂammable material close to the engine mufﬂer.

SAFETY

Use only authorized parts and accessories with your brushcutter. Do not make modiﬁcations to this unit without the written approval of , Inc.

ALWAYS make sure the cutting attach- ment is properly installed and ﬁrmly tightened before operation.

NEVER use a cracked or warped cutting attachment. If a properly installed attach- ment vibrates, replace the attachment with new one and re-check.

ALWAYS stop the engine immediately and check for damage if you strike a foreign object or if the unit becomes tangled. Do not operate with broken or damaged equipment.

NEVER allow the engine to operate at high RPM without a load. Doing so could damage the engine.

NEVER operate a unit with worn or dam- aged fasteners or attachment holders.

NEVER cut with a dull blade. Doing so will increase the risk of blade thrust and may also cause equipment damage.

BP3502A

BP3501A

SAFETY

The Properly Equipped Operator

Figure 1

Always make sure the appropriate cutting attachment shield is correctly installed and in good condition.

Wear appropriate footwear (non-skid boots or shoes): do not wear open-toed shoes or sandals. Never work barefooted!

Keep away from the rotating trimmer line or blade at all times,

and never lift a moving attachment above waist-high.

Always wear eye

protection such as goggles

or safety glasses to shield against thrown objects.

Always operate with both hands ﬁrmly gripping  the

unit.

Keep a proper footing and do not overreach. Maintain your balance at all times during operation.

Wear hearing protection devices and a broad-brimmed hat or helmet.

Wear close-ﬁtting clothing to protect legs and arms. Gloves offer added protection and are strongly recommended. Do not  wear clothing or jewelry that could get caught in machinery or under- brush. Secure long hair so

that it is above shoulder level. NEVER wear shorts!

Figure 2

If contact is made with a hard object, stop the engine and inspect the cutting attachment for damage.

Beware of a coasting blade when brushcutting. A coasting blade can injure while it continues to spin after the throttle trigger is released or after

the engine is stopped.

ALWAYS clear your work area of trash or hidden debris that could be thrown back at you or toward a bystander. When operating in rocky terrain or near electric wires or fences, use extreme caution to avoid contacting such items with the cutting attachment.

Be constantly alert for objects and debris that could be thrown either from the rotating cutting attachment or bounced from a hard surface.

Reduce the risk of bystanders being struck by ﬂying de- bris. Make sure no one is within 50 feet (15 meters)—that’s

about 16 paces—of an operating attach- ment.

Be extremely careful of slippery terrain, especially during rainy weather.

Make sure bystanders or observers outside the 50-foot “danger zone”

wear eye protection.

50

FEET

Avoid long-term operation in very hot or very cold weather.

Be Aware of the Working Environment

Figure 3

Cutting Attachment Shield

C35 BRUSHCUTTER

Throttle Trigger

Handle

Brushcutter Blade

Gearcase

Outer Tube

Ignition		Throttle Switch	Assembly

BP3503A

Mufﬂer

Spark Plug

Air Cleaner

Flex Cable

Fuel Tank

Cylinder Cover

Unit Description

DESCRIPTION

Using  the  accompanying  illustrations as a guide, familiarize yourself with this unit and its various components. Un- derstanding this unit helps ensure top performance, long service life, and safer operation.

WARNING!

Do not make unauthorized modiﬁca- tions or alterations to this unit or its components.

IMPORTANT!

BACKPACK brushcutter units do not have the spark arrester installed in the muf- ﬂer. Before placing the brushcutter in service, check local, state and federal regulations to determine if a spark arrester is required in your area. See page 15 for details.

# Speciﬁcations

Engine Model ....................................................... SK35F Engine

Dry Weight

(less cutting attachment and guard).................21.3 lbs./9.7 kg Type.................................... 2-cycle, vertical cylinder, air cooled Bore x Stroke........................................................36mm x 33mm

Displacement................................................... 33.6.cc/2.1 cu. in.

Maximum Power Output................ 1.8 hp @ 7500 RPM (min-1) Operating RPM Range......................... 4,500-7,000 RPM (min-1)

Transmission Type ...................... Automatic centrifugal-clutch

through bevel gears

Fuel/Oil  Ratio.............................................. 25:1

Premium 2-cycle mixing oil Fuel Tank Capacity ..........................................40.6 oz./ 1200 ml

Carburetion............................................................. TK,  ﬂoat  type

Ignition ........................... Fully electronic, transistor controlled Spark  Plug............................................................. Champion  CJ8

Air   Cleaner.............................................................Foam  element

Starting Method ..................................................... Recoil starter

Stopping Method................................................... Toggle switch

Speciﬁcations subject to change without notice

### Prior to Assembly

Before assembling this product, please make sure you have all the components required for a complete unit:

- Engine Assembly (Powerhead).

- Outer Tube Assembly.

- Cutting Attachment Shield.

- Handle.

- Tool bag including: Hex wrenches and a combination spark plug wrench and cutting attachment shield mount- ing hardware.

Carefully inspect all components for damage.

IMPORTANT!

The terms “left,” “left-hand,” and “LH”: “right,” “right-hand,” and “RH”; “front” and “rear” refer to directions as viewed by the operator during normal operation of this product.

BP3506

Figure 6

Flex Cable Housing

Groove

O-Ring

Rear Handle

BP35 Brushcutter

Release Knob Locking

Hole	“Engine Side”

Label

Throttle Cable

Cable Holder

Powerhead

Figure 4

Assembly: Driveshaft/Powerhead

7/8"

Figure 5

BP3505

ASSEMBLY

Place the powerhead on a ﬂat surface.

  - Route the “engine side” of the ﬂex cable through the cable holder.

  - Remove the two protective covers from the ﬂex cable and insert the ﬂex cable housing marked “engine end” into the powerhead with the locking hole facing up. See Figure 4.

  - Lift up on the ﬂex cable release knob and slide ﬂex cable housing all the way in until it bottoms in the case and the release knob locks into the lock- ing hole in the ﬂex cable housing. See Figure 5.

  - From the opposite end, rotate and push the drive cable to engage it into the clutch drum. See Figure 5.

BP3507

NOTE:

No more than 7/8” (22 mm) of the ﬂex cable should protrude from the ﬂex cable housing when properly installed.

  - Remove the Phillips head screw from the rear hand grip and slide the O-ring end of the ﬂex cable housing into the rear of the outer tube handle until the groove on the housing lines up with the hole for the Phillips head screw. See Figure 6.

IMPORTANT!

It may be necessary to rotate the output shaft of the gearcase to fully engage the drive cable into the lower tube assembly.

  - Install and tighten the Phillips head screw ﬁrmly.

Throttle and Handle Assembly

BP3510

BP3508

Cable Guide

Figure 9

Install the Throttle Cable

Figure 11

Lower Clamp

Outer Tube

Handle Position Label

Clamp Retaining   	 Screw

Install the Handle

Cable Guide

Throttle Lever

Figure 7

Loosen screws

    - Loosen the two screws securing the throttle assembly and slide it forward on the outer tube. See Figure 7.

BP3509

Cable End

Throttle Cable

Figure 8

Remove the cable guide to allow the throttle trigger to swing out. See Figure 7.

    - Slide the throttle cable through the hand grip and insert the cable end into the recess in the throttle trigger. See Figure 8.

    - Install the cable guide and slide the throttle assembly back into place. Tighten the screws securely. See Figure 9.

BP3511

Figure 10

Connect the powerhead and stop switch wire terminals.

BP3504

Use a Phillips head screwdriver to remove the lower clamp retaining screws from the handle. Separate the clamp from the handle.

      - Position the handle on the outer tube behind the handle positioning label. Reassemble the lower clamp to the handle in the reverse order of disas- sembly. See Figure 11.

      - Firmly tighten the (4) handle retain- ing screws. See Figure 11.

Adjust Throttle Cable Free Play

BP3514

Checking Free Play	Adjusting Free Play

7mm Free Play

Figure 12

BP35 Brushcutter

Cable Adjuster

Cable Locknut

Adjuster Cover

Figure 13

        - Test the throttle lever for smooth op- eration. If any stiffness or binding are noted, the cause must be identiﬁed and corrected before the brushcutter can be placed in service.

        - Test the throttle lever for proper “free play” of approximately 7mm in the idle position. See Figure 12. If neces- sary, adjustments can be made at the carburetor.

- Temporarily move the adjuster cover to expose the cable adjuster.

- Loosen the cable locknut and then turn the cable adjuster in or out until proper free play is achieved.

See Figure 13.

- Tighten cable adjuster locknut and move adjuster cover back into place.

Install the Cutting Attachment Shield

WARNING!

NEVER operate the unit without the cutting attachment shield installed and tightly secured!

Shim

Cutting Attachment Shield Mounting Plate

Cutting Attachment Shield

Figure 14

Nut

Figure 14A

Hex Screws

Shim

Line Cutter

Outer Tube

Socket-head Cap Screws

Bracket

Sub-Shield.

(when trimmer head is in use)

WARNING!

NEVER use this machine without

sub-shield when using a trimmer head.

Attach the shield extension to the cutting attachment shield.

### Install the Cutting Attachment Shield.

- Insert the cutting attachment shield between the outer tube and the cutting attachment shield mounting plate. See Figure 14.

NOTE:

It may be necessary to loosen the retain- ing nut and clamp screw to adjust cutting attachment shield mounting plate.

- Fit the two shims and the bracket over the outer tube and loosely install the four socket-head cap screws. See Figure 14.

CAUTION!

Make sure the clamp screw and retaining nut are securely tightened before tightening the four socket- head cap screws.

Cutting Attachment Shield

Hook Receiver

Hook

Sub-shield

Tighten the four socket-head cap screws to secure the cutting attach- ment shield.

CAUTION!

Make sure the sub-guard is com- pletely hooked at the hook receiver.

ASSEMBLY

To Change Position of Line Cutter.

The line cutter can be positioned in

2 positions to obtain different line length for cutting.

- Remove the 2 hex screws with a 4mm hex wrench. See Figure 14A.

- Rotate line cutter. See Figure 14A.

- Reinstall the two hex screws and tighten them securely.

WARNING!

The line cutter is very sharp. Wear gloves to protect your hands when handling.

NOTE:

Be careful to not lose the 2 nuts in the cutting attachment shield, they are not captured.

Figure 15

Slip the blade into place

B

Output Shaft

Hex Wrench

Shaft Bolt

Bolt Guard Holder B

Safety Clip

Holder A

C

Center the safety clip

A

Slide the safety clip off-center

Assembly

### Install the Blade (BACKPACK)

NOTE:

The BACKPACK is shipped with Holder A, the  safety clip, Holder B, shaft bolt, and bolt guard installed. The shaft bolt is a LEFT- HAND THREAD AND IS REMOVED IN A CLOCKWISE ROTATION!

35011

35010

ASSEMBLY

With the gearcase output shaft fac- ing up, rotate the output shaft and Holder A until the hole in Holder  A aligns with the matching hole in the gearcase ﬂange, and then lock the holder to the gearcase by insert- ing the long end of the hex wrench through both holes. See Figure 15.

- Remove the shaft bolt, bolt guard and Holder B. See Figure 15.

- Slide the safety clip off center on the gearcase shaft. See Figure 15A.

NOTE:

When installing certain blades, it may be necessary to temporarily remove the

safety clip.

35012

35013

Slide the blade over the safety clip and onto the ﬂange on Holder A. See Figure 15B.

Figure 16

Cutting Blade

Install Holder B

35014

Lock the blade on the shaft by centering the  safety  clip. See Figure 15C.

CAUTION!

Install the blade so the printed surface is visible to the operator while the brushcutter is in the normal operating position.

- Install Holder B on the gearcase shaft. See Figure 16.

WARNING!

Never operate the brushcutter without the safety clip installed and both holders tightly secured and ﬂat against the blade surface!

Tighten the assembly

Combination Spark Plug/Screwdriver Wrench

Holder B

Bolt Guard

Hex Wrench

Figure 17

IMPORTANT!

35015

The machined recess in Holder B must completely surround the safety clip, and both holders must be ﬂat against the surface of the blade.

- Lock Holder A to the gearcase by inserting the long end of the hex wrench through both holes as done in step 1 and tighten the shaft bolt securely with the combination spark plug wrench. See Figure 17.

The BP35 should now be completely assembled.

Remove the hex wrench.

Assembly

### Installing a Trimmer Head

NOTE:

The BACKPACK is shipped with Holder A, the blade retainer (safety clip), Holder B, shaft bolt, and bolt guard installed. The shaft bolt is a LEFT-HAND thread. Remove it by turning CLOCKWISE!

- Remove the shaft bolt, bolt guard, Holder B, safety clip and the existing cutting attachment.

- With the gearcase output shaft fac- ing up, rotate the output shaft and Holder A until the hole in Holder  A aligns with the matching hole in the gearcase ﬂange, and then lock the holder to the gearcase by insert- ing the long end of the hex wrench through both holes. See Figure 18A.

- Using the combination spark plug wrench, remove the shaft bolt, bolt guard, Holder B and the safety clip. (The bolt guard, shaft bolt and safety clip are not used with a trimmer head). See Figure 18A.

35008

ASSEMBLY

| A | Shaft Bolt | B | Hand-tighten Trimmer Head |
| --- | --- | --- | --- |
| Bolt Guard (not used) (not used) Holder B Safety Clip (not used) Holder A 35007 Output Shaft Figure 18 |  | (counter-clockwise to install) Hex Wrench |  |

Install Holder B on the gearcase shaft. The splined hole on Holder A must engage with the gearcase shaft.

- Using the hex wrench to secure  Holder A , install and hand-tighten the trimmer head (counter-clockwise to install). See Figure 18B.

- Remove the hex wrench from the gearcase and holder.

CAUTION!

- Some gasolines contain alcohol as an oxygenate! Oxygenated fuels may cause increased oper- ating temperatures. Under certain conditions, alcohol-based fuels may also reduce the lubricating qualities of some mixing oils. Nev- er use any fuel containing more than 10% alcohol by volume!

- Generic oils and some outboard motor oils may not be intended for use in high-performance air cooled 2-cycle engines, and should never be used in your Shindaiwa engine!

Mixing Fuel

  - Use only fresh, clean unleaded gaso- line with a pump octane rating of 87 or higher.

  - Mix gasoline with 50:1 Premium 2-cycle mixing oil or with an equivalent high quality 2-cycle mixing oil.

##### Example of 50:1 mixing quantities:

  - 1 gallon of gasoline to 2.6 oz. mixing oil.

  - 5 liters of gasoline to 100 ml. mixing oil.

IMPORTANT!

CAUTION!

This engine is designed to oper- ate on a 50:1 mixture consisting of unleaded gasoline and a premium 2-cycle mixing oil only. Use of non- approved mixing oils can lead to

excessive maintenance costs and/or engine damage.

Mix only enough fuel for your immedi- ate needs! If fuel must be stored longer than 30 days and One oil with fuel stabilizer is not used, it should ﬁrst be treated with a fuel stabilizer such as STA-BIL™.

Filling the Fuel Tank

WARNING!

Minimize the Risk of Fire!

- STOP the engine before refueling.

- ALWAYS allow the unit to cool before refueling!

- Wipe all spilled fuel and move the unit at least 10 feet (3 me- ters) from the fueling point before restarting!

- NEVER start or operate this unit if there is a fuel leak.

- NEVER start or operate this unit if the carburetor, fuel lines, fuel tank and/or fuel tank cap are damaged.

- NEVER smoke or light any ﬁres near the unit or fuels!

- NEVER place any ﬂammable material near the engine mufﬂer!

- NEVER operate the engine with- out the mufﬂer and spark arrester in place and properly functioning!

Place the unit on a ﬂat, level surface.

- Clear any dirt or other debris from around the fuel ﬁller cap.

- Remove the fuel cap, and ﬁll the tank with clean, fresh fuel.

- Reinstall the fuel ﬁller cap and tighten ﬁrmly.

Ignition ON

Figure 19

Starting the Engine

Choke Closed

Figure 22

While holding the unit ﬁrmly with your left hand, use your right hand to pull the starter handle slowly upward until you feel the starter engage. See Figure 23.

- Start the trimmer by pulling the starter handle upward rapidly.

BP3521

BP3522

1. Move the ignition switch to the “ON” position.

Figure 20

Throttle Lever

Lock Button

BP35 Throttle Assembly

CAUTION!

The recoil starter can be easily dam- aged by abuse!

- Always engage the starter before attempting to start the engine.

- Never pull the starter cord to its full length.

- Always rewind the starter cord slowly.

(Cold Engine Only) Choke the engine by moving the choke lever up towards the spark plug (choke is closed). See Figure 22.

BP3520

Choke Open

Figure 24

WARNING!

KEEP WELL CLEAR OF THE CUT- TING ATTACHMENT! THE CUT- TING ATTACHMENT MAY ROTATE WHEN THE ENGINE IS STARTED!

- Place the trimmer on the ground during all starting operations.

- Make sure you have a secure footing, and keep a ﬁrm grip on the unit as well.

- Keep all bystanders and pets well clear of the brushcutter during starting.

OPERATION

When the engine starts or ﬁres–

  - Set the throttle lever to “fast idle” by performing the following:

    - Squeeze and hold throttle lever (toward the handgrip).

    - Depress and hold throttle lever lock button.

Figure 21	Fuel Cock Open

While holding the throttle lever lock button, release throttle lever.

BP3524

Open the choke by moving the choke lever down. See Figure 24.

Make sure the cutting attachment is clear of obstructions!

Figure 23

If the engine does not continue to run, repeat the starting procedure as previously described.

BP3523

When the engine starts, clear excess fuel from the combustion area by rev- ving the engine several times with the throttle lever.

WARNING!

The cutting attachment will rotate as the engine accelerates!

BP3521

3. Open the fuel cock by moving the lever to the down position See Figure 21.

# Starting a Flooded Engine

- Disconnect the spark  plug  lead, and then use the spark plug wrench to remove the spark plug (turn counter-clockwise   to   remove). See Figure 25.

- If the spark plug is fouled or is soaked with fuel, clean or replace the plug as necessary. For spark plug speciﬁca- tions and gapping procedure, see page 14.

- Pull the starter rope several times with the choke open and at wide open throttle to clear excess fuel from the combustion chamber.

10. Operating the throttle will automatic- ally disengage the fast-idle setting.

WARNING!

Never start engine from the operating position!

If the engine does not start–

Turn counter- clockwise to remove

Spark Plug

Figure 25

Repeat the appropriate starting procedures for a hot or cold engine. If the engine still fails to start, use the following procedure for “Starting a Flooded Engine”.

- Replace the spark plug and tighten it ﬁrmly with the spark plug wrench. If  a torque wrench is available, torque the spark plug to 148-165 inch-pounds (170-190 kg/cm).

CAUTION!

Incorrect spark plug installation can result in serious engine damage!

- Repeat the starting procedures for a warm engine.

3526

If the engine still fails to start or ﬁre, refer to the troubleshooting chart at the end of this manual.

# Stopping the Engine

# Engine Idle Adjustment

BP3525

Idle Adjusting Screw

Figure 27

Ignition OFF

Figure 26

WARNING!

The cutting attachment must NEVER rotate at engine idle! If the idle speed cannot be adjusted by the procedure described here, have the unit in- spected at an authorized Shindaiwa dealer.

The engine must return to idle speed whenever the throttle lever is released. Idle speed is adjustable, and must be set low enough to permit the engine clutch to disengage the cutting attachment when the throttle is released.

BP3526

Idle the engine brieﬂy before stopping (about 2 minutes), then push the ignition switch to the off position to stop the engine. See Figure 26.

- Place the unit on the ground, then start the engine and allow it to idle 2-3 minutes until warm.

- If the attachment rotates when the engine is at idle, reduce the idle speed by turning the idle adjustment screw counter-clockwise

See Figure 27.

OPERATION

If a tachometer is available, the engine idle speed should be ﬁnal adjusted to 2,750 (±250) RPM (min-1).

Left Hand Operation

Some users of the BACKPACK may wish to operate the unit left handed. It is very important to follow these instructions if you wish to do so.

  - Loosen the exhaust outlet clamp and rotate the exhaust outlet approxi- mately 90 degrees counterclockwise as viewed from the top of the power head.

  - Tighten the clamp screw ﬁrmly to prevent the outlet from moving out of position.

BP3527

RH Operation

LH Operation

Figure 28

Exhaust Outlet

Clamp Screw

Put unit on, and in the operating posi- tion, check exhaust outlet to make sure exhaust ﬂow is out and to the side of the operator. Readjust if neces- sary. See Figure 28.

Figure 29

B

A

Shoulder Straps

### Adjust the Shoulder Straps:

    - Place the Brushcutter on your back with the straps loosened.

    - To tighten straps,  push  the  outer strap through the buckle from the top until the loop formed is approximately the amount of adjustment necessary. See Figure 29A.

    - While holding the lower outer strap, grasp the buckle and pull upward until the strap is tight in the buckle. See Figure 29B.

    - To loosen, Reverse this procedure.

# Using a Brushcutter Blade

OPERATION

Blade Thrust

‘Blade thrust’ is a sudden sideways or backward motion of the brushcutter. Such motion may occur when the blade jams or catches on an object such

as a sapling tree or tree stump. BE CONSTANTLY ALERT FOR BLADE THRUST AND GUARD AGAINST ITS EFFECTS!

### Engine Operating Speeds

WARNING!

- Before working with a blade- equipped unit, always inspect and clean the area of objects that

could interfere with or damage the blade.

- Never use a blade near side- walks, fence posts, buildings or other objects that could cause injury or damage.

- Never use a blade for purposes other than those for which it was designed.

- Whenever you strike a hard object with a blade, always stop the brushcutter and carefully inspect the blade for damage. NEVER OPERATE THE BRUSHCUTTER WITH A DAMAGED BLADE!

- Always make sure the cutting attachment shield is properly installed before operating the unit.

Operate the engine at full throttle while cutting. Best fuel efﬁciency is obtained by releasing the throttle when swinging back after a cut.

- To prevent possible engine damage, do not allow the brushcutter to run at high speeds without a load.

Cut on the left side of the blade. KEEP YOUR BODY OUTSIDE THE PATH OF BLADE ROTATION

Figure 31

Avoid operating the attachment at low speeds. Doing so can lead to rapid clutch wear. In addition, slow-speed operation tends to cause grass and debris to wrap around the cutting at- tachment.

The blade rotates counter-clockwise. For best performance and to minimize being struck by debris, move the blade from right to left while advancing on your work.

BP3531A

Ten O'clock

Blade Rotation

OK To Cut

Eight O'clock

Five O'clock

Figure 30

Position the blade so cuts are made between the blade’s 8 o’clock and 10 o’clock positions (as viewed from above). DO NOT cut between the 10 o’clock and 5 o’clock positions (shaded area). See Figure 30.

### Vertical cuts

Hold the brushcutter with the blade at  a 90° angle to the ground so the blade’s bottom edge rotates toward the opera- tor. Move the blade from top to bottom through the cut, and cut only with the bottom edge of the blade. See Figure 31.

WARNING!

When making vertical cuts, never allow the blade to exceed waist height.

WARNING!

When cutting wood with a blade, feed the blade slowly. Never strike or “slam” a spinning blade against the wood.

WARNING!

DO NOT use 2-tooth or NON- Shindaiwa approved 4-tooth cutting blades with Shindaiwa brushcutters.

Cutting Grass

Your BACKPACK Brushcutter may be ﬁtted with one of several trimmer head models, each with features for speciﬁc applications and/or opera- tional requirements.

For proper operation, always refer to the instructions accompanying the trimmer head being used. Available trimmer head styles include:

  - Semi-automatic. Trimmer line is indexed when the operator taps the trimmer head on the ground during operation.

  - Manual. The operator indexes line manually with the grass trimmer stopped.

  - Fixed. The operator must stop the unit and add new lengths of trimmer line manually.

  - Flail. This device, designed for clear- ing weeds and light brush, features three nylon blades attached to the head by pivots.

CAUTION!

Do not push the rotating line into trees, wire fences or any material that could tangle or break line ends.

### Engine Operating Speeds

Operate at full throttle while cutting grass.

CAUTION!

Operation at low RPM can lead to premature clutch failure.

BP3529A

Figure 32

Hold the unit so the trimmer head is angled slightly into the area to be cut. To ensure maximum trimmer-line service life, cut only with the tip of the trimmer line. To prevent grass clippings from being thrown back towards the operator or wrapping around the gearcase, swing the unit’s trimmer head from left to right while cutting. Keep the trimmer head horizontal. See Figure 32.

### Edging

Figure 33

Tilt the handle about 100° to the left (from hori- zontal) and move forward, holding the unit so that the trimmer head cuts vertically.

OPERATION

See Figure 33.

WARNING!

- Never repair a damaged blade by welding, straightening, or by modifying its shape. An altered blade may break during opera- tion, resulting in serious personal injury.

- DO NOT use 2-tooth or NON- Shindaiwa approved 4-tooth cutting blades on Shindaiwa trimmers or brushcutters.

- Blades are not interchangeable between Shindaiwa edgers and trimmer/brushcutter models. Operating any unit with a blade or attachment not approved for that unit can be hazardous and may cause serious injury.

WARNING!

Before performing any maintenance, repair or cleaning work on the unit, make sure the engine and cutting attachment are completely stopped. Disconnect the spark plug wire before performing service or mainte- nance work.

General Maintenance

### Mufﬂer

Make sure the mufﬂer is well secured and in good condition. A worn or dam- aged mufﬂer is a ﬁre hazard and may also cause hearing loss.

### Spark Plug

Keep the spark plug and wire connec- tions tight and clean.

### Fasteners

WARNING!

Non-standard parts may not oper- ate properly with your unit and may cause damage and lead to personal injury.

Make sure nuts, bolts, and screws (except carburetor adjusting screws) are tight.

### Blades

Keep blades sharp and check blade condition frequently. If a blade’s per- formance changes suddenly, stop the

NOTE:

Using non-standard replacement parts could invalidate your warranty.

engine and check the blade for cracks or other damage. Replace a damaged blade IMMEDIATELY!

MAINTENANCE

Daily Maintenance

##### Prior to each work day, perform the following:

  - Remove all dirt and debris from the engine, check the cooling ﬁns and air cleaner for clogging, and clean as necessary.

- Carefully remove any accumulations of dirt or debris from the mufﬂer and fuel tank. Dirt build-up in these areas can lead to engine overheating, ﬁre, or premature wear.

- Check for loose or missing screws or components. Make sure the cutting attachment is securely fastened.

- Check the entire unit for leaking fuel or grease.

10-Hour Maintenance

Every 10 hours of operation (more frequently in dusty or dirty conditions):

Clip

Figure 34

Inner Screen

Outer Screen

Air Cleaner	Remove (3) Element		Screws

CAUTION!

Do not operate the unit if the air cleaner or element is damaged, or if the element is wet.

Remove the air cleaner element from the carburetor and clean it thor- oughly with soap and water. Let dry before reinstalling  the  element. See Figure 34.

Figure 35

Clean the spark plug and check the gap at the electrode.

0.024-inch (0.6 mm)

Turn counter- clockwise to remove

Spark Plug

10/15-Hour Maintenance

##### Every 10 to 15 hours of operation:

Remove and clean the spark plug.

3535

3526

Adjust the spark plug electrode gap to 0.024-inch (0.6 mm). If the plug must be replaced, use only a Champion CJ8 or equivalent spark plug of the cor- rect heat range. See Figure 35.

CAUTION!

Before removing the spark plug, clean the area around the plug to prevent dirt and debris from getting into the engine’s internal parts.

Cable (clean and grease)

Shaft Tube

Figure 38

Fuel Filter

Figure 37

50-Hour Maintenance

Every 50 hours of operation (more frequently in dusty or dirty conditions):

BP3537

Figure 36

Gear Shaft Collar

Old Grease

New Grease

Remove and clean the cylinder cover and clean grass and dirt from the cylinder ﬁns.

BP3535

Remove the cutting attachment, holder and the gear shaft collar. Remove the ﬁller plug from the side of the gearcase and press new grease into the gearcase until the old grease has been pushed out. Use only lith- ium-base grease such as Gearcase Lubricant or equivalent. See Figure 36.

- Pull apart the fuel ﬁlter hose at the joint and remove the fuel ﬁlter to clean or replace as necessary See Figure 37.

  - Lubricate the ﬂex cable as follows:

- Disconnect the ﬂex cable from the powerhead. Slide the ﬂexible cable out of the ﬂex cable housing.

- Wash the cable thoroughly in a solvent bath, and then coat the entire cable with Premium Gearcase Lubricant.

- Install the cable into the ﬂex cable housing, then reconnect the ﬂex cable assembly to the powerhead.

NOTE:

MAINTENANCE

For maximum cable life, the ﬂexible cable should be reversed “end-for-end” during reinstallation.

# Spark Arrester

### Installation

IMPORTANT!

BACKPACK brushcutter units do not have the spark arrester installed in the muf- ﬂer. Before placing the brushcutter in service, check local, state and federal regulations to determine if a spark ar- rester is required in your area.

  - Obtain a spark arrester and gasket from an authorized dealer.

  - Remove the (3) socket-head cap screws retaining the mufﬂer cowl and remove the cowl. See Figure 39.

  - Remove the spark arrester outlet cap by removing the (2) outlet cap screws.

3538

Install the spark arrester gaskets, screen and cap. Tighten the (2)

Outlet Cap Screws

Outlet Cap

Figure 39

Gasket

Mufﬂer Cowl (secured by 3 socket-head cap screws)

Mufﬂer

Spark Arrester Screen

Socket-head Cap Screws

Mufﬂer Maintenance

outlet cap screws securely. See Figure 39.

  - Reinstall the mufﬂer cowl and tighten the (3) socket-head cap screws securely.

If the engine becomes hard to start or

has low power, the spark arrester screen should be inspected and cleaned.

WARNING!

Never operate the unit with a dam- aged or missing mufﬂer! Operating with missing or damaged exhaust components is a ﬁre hazard and could also damage your hearing.

Remove the (3) socket-head cap screws retaining the mufﬂer cowl and remove the cowl. See Figure 39.

- Remove the spark arrester screen by removing the (2) screws to the exhaust outlet cap.

- Remove the spark arrester screen and clean with a stiff bristle brush.

- Reinstall the spark arrester screen

and cap. Tighten the (2) screws securely.

- Reinstall the mufﬂer cowl and tighten the (3) socket-head cap screws securely.

IMPORTANT!

If carbon deposits are severe or if no performance improvement is noted, this unit should be inspected by an autho- rized servicing dealer.

# Long Term Storage

Whenever the unit will not be used for 30 days or longer, use the following pro- cedures to prepare it for storage:

- Clean external parts thoroughly.

- Drain all the fuel from the fuel tank.

IMPORTANT!

All stored fuels should be stabilized with a fuel stabilizer such as STA-BIL™.

To remove the remaining fuel from the fuel lines and carburetor and with the fuel drained from the fuel tank;

- Pull apart the fuel ﬁlter hose at the joint and drain all fuel into an ap- proved fuel container. See page 15 (fuel ﬁlter replacement).

- Start and run the engine until it stops running.

- Repeat steps 1 and 2 until the engine will no longer start.

- Remove the spark plug and pour about 1/4 ounce of 2-cycle mixing oil into the cylinder through the spark plug hole. Slowly pull the recoil starter 2 or 3 times so oil will evenly coat the interior of the engine. Rein- stall the spark plug.

CAUTION!

Gasoline stored in the carburetor for extended periods can cause hard starting, and could also lead to increased service and mainte- nance costs.

Before storing the unit, repair or replace any worn or damaged parts.

- Remove the air cleaner element from the carburetor and clean it thoroughly with soap and water, let dry and reas- semble the element.

- Store the unit in a clean, dust-free area.

# Blade Sharpening

MAINTENANCE

When the cutting edges of the blade become dull, they can be resharpened with a few strokes of a ﬁle.

In order to keep the blade in balance, all cutting edges must be sharpened equally.

### Tornado™ Blade

Round File

30°

Figure 40

To sharpen the cutters on a Tornado Blade, use a 7/32-inch round ﬁle. File

WARNING!

Sharpen only the cutting teeth of a blade. DO NOT alter the contour of the blade in any way.

the leading edge of each tooth to a razor edge. The top plate of each tooth should angle back 30°. See Figure 40.

### Multiple-tooth Circular Blade

Use a round ﬁle to maintain a radius of

Round File

Figure 41

0.04 to 0.06” (1 to 1.5 mm) at the base of each tooth. Cutting edges must be offset equally on each side. See Figure 41.

# Troubleshooting Guide

ENGINE DOES NOT START

NO

Does the engine crank?

What To Check	Possible Cause	Remedy

Faulty recoil starter.

NO

Good compression?

YES

NO

Does the tank contain fresh fuel of the proper grade?

YES

NO

Is fuel visible and moving in the return line when priming?

| Is there spark at the spark plug wire terminal? |  | NO |
| --- | --- | --- |
| YES |  |  |

YES  YES

Fluid in the crankcase Internal damage.

Loose          spark           plug. Excess wear on cylinder, piston, rings.

Fuel incorrect, stale, or contaminated; mixture incorrect.

Check for clogged fuel ﬁlter and/or vent.

The ignition switch is in “O” (OFF) position. Shorted ignition system.

Faulty ignition unit.

Check the spark plug.

If the plug is wet, excess fuel may be in the cylinder.

The plug is fouled or improperly gapped.

The plug is damaged internally or of the wrong size.

Consult with an authorized servicing dealer.

Tighten                and                re-test. Consult with an authorized servicing dealer.

Reﬁll with clean, fresh unleaded gasoline with a pump octane of 87 or higher, mixed with Premium 2-cycle mixing oil at a 50:1 gasoline/oil ratio.

Replace fuel ﬁlter or vent as required. Restart.

Move switch to “I” (ON) position and re-start. Consult with an authorized servicing dealer.

Crank the engine with the plug removed, replace the plug, and restart.

Clean and gap the spark plug to 0.024 inch (0.6mm). Restart.

Replace the plug with a Champion CJ8 or equivalent type spark plug of the correct heat range. Restart.

LOW POWER OUTPUT

## What To Check	Possible Cause	Remedy

Is the engine overheating?

TROUBLESHOOTING

Operator is overworking the machine. Carburetor mixture is too lean.

Improper fuel ratio.

Engine is rough at all speeds. May also have black smoke and/or  unburned fuel at the exhaust.

Fan, fan cover, cylinder ﬁns dirty or damaged. Carbon deposits on the piston or in the mufﬂer. Clogged air cleaner.

Loose or damaged spark plug.

Air leakage or clogged fuel line. Water in the fuel.

Engine is knocking.

Piston                         seizure. Faulty carburetor and/or diaphragm. Overheating condition.

Improper fuel.

Carbon deposits in the combustion chamber.

Shorten trimmer line. Cut at a slower rate. Consult with an authorized servicing dealer.

Reﬁll with clean, fresh unleaded gasoline with a pump octane of 87 or higher, mixed with Premium 2-cycle mixing oil at a 50:1 gasoline/oil ratio.

Consult with an authorized servicing dealer. Clean or replace the air ﬁlter.

Tighten or replace the plug with a Champion CJ8 or equivalent type spark plug of the correct heat range. Restart.

Repair or replace fuel ﬁlter and/or fuel line. Reﬁll with fresh fuel/oil mixture. See page 9.

Consult with an authorized servicing dealer.

See above.

Check fuel octane rating; check for presence of alcohol in the fuel. See page 9.

Refuel as necessary.

Consult with an authorized servicing dealer.

Troubleshooting Guide (continued)

ADDITIONAL PROBLEMS

## Symptom	Possible Cause	Remedy

Poor acceleration.

Engine stops abruptly.

Clogged air ﬁlter. Clogged fuel ﬁlter. Lean fuel/air mixture. Idle speed set too low. Switch turned off.

Fuel          tank          empty. Clogged          fuel          ﬁlter. Water      in       the       fuel. Shorted spark plug or loose terminal.

Ignition failure. Piston seizure.

Engine difftcult to shut off.

Ground (stop) wire is disconnected, or switch is defective.

Overheating due to incorrect spark plug.

Cutting attachment rotates at engine idle.

TROUBLESHOOTING

Overheated engine. Engine idle too high.

Broken clutch spring or worn clutch spring boss. Loose attachment holder.

Excessive vibration.

Warped or damaged cutting attachment. Loose gearcase.

Cutting attachment will not rotate.

Bent main shaft/worn or damaged bushings. Shaft not installed in powerhead or gearcase. Broken shaft.

Damaged gearcase.

Clean or replace the air ﬁlter. Replace the fuel ﬁlter.

Consult with an authorized servicing dealer. Adjust: 2,750 (±250) RPM (min-1).

Reset the switch and restart. Refuel. See page 9.

Replace                                 ﬁlter. Drain; replace with clean fuel. See page 9.

Clean or replace spark plug with a Champion CJ8 or equivalent type spark plug of the correct heat range. Restart. Tighten the terminal.

Replace      the      ignition      unit. Consult with an authorized servicing dealer. Test and replace as required.

Replace spark plug with a Champion CJ8 or equivalent type spark plug of the correct heat range.

Idle              engine               until               cool. Set   idle:   2,750   (±250)   RPM    (min-1). Replace spring/shoes as required, check idle speed. Inspect and re-tighten holders securely.

Inspect and replace attachment as required. Tighten gearcase securely.

Inspect and replace as necessary. Inspect and reinstall as required.

Consult with an authorized servicing dealer.

NOTES: