433 wireless         Barcode Scanner     Manual

Specifications：

Scanner technical parameters

Data storage: 512K, inventory model can store 3000 bar code

Wireless standard: AFH adaptive frequency hopping

Interface: compatible with USB3.0

Communication range: 300 meters (open range of visibility)

Keyboard layout: United States (default)

Light source: 650 nm visible laser

Scanning speed: 200 times per second

Decoding ability: all one dimensional bar code

Input voltage: scanner charging power supply:

DC:+4.75V ~ +5.25V

Receiver: DC:+4.75V+5.25V USB

Battery: 1100Mah

Working current: sleep: 0.5MA: scan: 65MA:

Data transmission: 135MA

Working hours: >20 hours

Tips: buzzer, LED

Working temperature: 0 degrees C to 50 degrees C

Storage: -40 degrees C to 60 degrees C

Humidity: 5% to 95%

Safety regulations: EN60825-1 1, First class laser safety

standards

Electromagnetic compatibility: EN 55022, EN55024

Electrical safety:  EN 60950-1

Environmental protection: with the ROHS standard

Material: ABS+TPU

Net weight: 150g

System function setting

Factory Default setting

All scanners have a factory default settings, scan "Factory Default "bar code, will make the scanner all property settings to be factory default setting.

In the following cases you can use this bar code:

1. The scanner setting error, such as unable to read bar code.

2. You have forgotten what you did before to the scanner settings, and do not want to be affected by the previous settings.

3.Setting the scanner to use a function that is not frequently-used, and the use of after.

Factory Default

System function setting

Wireless function settings

Pairing setting code

When the barcode scanner and the Bluetooth device can not transmit data, please scan this bar code.This operation will delete the original pairing product.

Note: when the barcode scanner to search the Bluetooth devices, the barcode scanner must work under the Wireless mode . (function setting please see table below)

Wireless mode        automatic storage mode

Internal storage mode       displays total storage

Clear all data                upload data

System function setting

function setting

Display version                Display factory date

key trigger mode          Automatic continuous scanning

Open sound                     Turn sound off

All for Capital           All for lowercase letter

Positive image recognition      Reverse  image recognition

System function setting

prefix or suffix  setting

Hide prefix 5 characters             Hide prefix space

Hide Suffix 5 characters             Hide Suffix 1 characters

Suffix add Enter                  Suffix add Enter and Tab

Prefix add 【STX】guide            Suffix add【ETX]

Cancel hidden prefix characters   Cancel hidden suffix characters